Lab Jaw Crusher Instructions

ShiCheng Oasis Mineral Equipment Manufacturing Co., LTD

Address: Industrial Park, Shicheng County, Ganzhou City, Jiangxi Province, China

Email:kevinsong@stejx.com                      Mob:+86-18389228612

LZPE series Jaw Crushers

This instruction is supplied to the operators to have acquaintance with PE series Jaw Crushers structure principles and manipulating methods ，please read it carefully before you manipulating.

- Purpose and Applied field

Our Jaw Crushers are ideally suitable for primary and secondary crushing, with well-capacity and high-efficiency, they are widely used for crushing various medium hard rocks and other ore materials whose maximum pressure-withstanding is 2000 kilogram force/cm3 and they are mainly supplied to the departments like Geological coal char、Mine、Metallurgy、Chemical Engineering、Construction Materials etc. And the labs of junior college academies science and research units.

Primary specifications and Technical data

| Model | Input opening | Input size | Output size | Capacity | Motor |
| --- | --- | --- | --- | --- | --- |
| LZPE60×100 | 60×100mm | ≤55mm | 3-10mm | 230-400kg/h | 1.5kw |
| LZPE100×150 | 100×150mm | ≤100mm | 6-38mm | 500-2000kg/h | 3kw |
| LZPE100×100 | 100×100mm | ≤80mm | 3-25mm | 400-1800kg/h | 3kw |
| LZPE150×250 | 150×250mm | ≤125mm | 10-40mm | 700-5000kg/h | 3kw |

Main working principled

The Jaw crusher structure includes Fabricated Body、Swing Jaw、Eccentric Shaft、Jaw Plates、Toggle mechanism etc parts. The electromotor drives eccentric shaft through triangle strap, to make the swing jaw moves in adjusted locus, thereby the materials in the crushing cavity are crushed.

The fabricated body is electricity welded by steel plates, the fixed jaw plates tightened by bolts are fixed in the front of steel housing, which assist in impacting the plates. The side plates are fixed in the steel housing side-wall which in the right-and-left of the crushing cavity, for preventing the wearing and tearing of the steel housing side-wall.

The swing jaw is a cast steel moulding, its face fixes swing jaw plates, top hang on the housing by eccentric shafts and roller bearings, its infrastructure is supported by toggle mechanism and it is scroll touching with the toggle mechanism. Fly wheels and Strap wheels with grooves are fixed in the two sides of the eccentric steel shafts. The steel housing is welded into the machine seat, electric motor sliding-track is fixed in the machine seat with bolts and a outlet drawer is fisted in the outlet, but it can expel products downwards directly without outlet drawer.

Toggle mechanism is connected with supporting bolts byφ9mm pins when the materials in the crushing cavity can not be crushed and the loads are increased suddenly, viz. The pin parts itself rapidly to protect other works from being destroyed. In the end of supporting bolt, there’s a hand wheel, it is used to adjust the outlet width size. The adjusting process is that remove the setscrew first, then circumgyrate the hand wheel, after adjusting, tighten the setscrew again, the connection between pothook and spring at the down most of the swing jaw make the toggle mechanism and swing jaw not to work outside, and they are adjusted by nuts.

Installation instruction

- The machine is supplied to users after being assembled whole by our manufacture factory, please examine it when you get ,in order to eliminate the possible faults during transporting.

- Due to biggish working vibrancy, so we suggest to mount the jaw crushers on the concrete foundations, for reducing vibrations and noises, you’d better underlay hardwood temples, rubber strips or other cushioning- vibrancy materials between the crushers and the concrete foundations.

- The foundation weight is approximately 5-10 times of the machine weight, the foundation depth is deeper than the ground freezing depth of this place .With regard to the machine installation and terra feet bolts location.

- Before adjusting the outlet width size, you should loosen the tense spring first. And after adjusting, appropriately adjust the spring tense degree, for preventing the toggle mechanism from working outside.

- Operation Notices

Starting preparations

- Carefully examine whether the bearings lubricating is well, whether the joints in the bearings or toggle mechanism have enough lubricating grease.

- Carefully examine whether all the tightening parts are tightened absolutely.

- Carefully examine whether transmission strips are all right, if not, replace the broken strips in time. When the strips or the strip wheels are smeary, please use clean rag to clean it.

- Examine whether there are rocks or other sundries in the crushing cavity, if are, please clean out them.

Jaw crushers jump-starting

- After examining, when the machine and the transmission parts are proved to be all up to snuff, then you can start it.

- This machine only permits starting without loads.

- After starting, if finding anything abnormal, please stop starting at once, examine and eliminate the abnormalities, then you can restart the crushers.

Maintaining and Using

- Only when the crushers are working normally, you can begin to feed the materials.

- The materials being crushed should be fed into the crushing cavity equably, and you should avoid feed in the side face; for preventing the loads break and unilateral sudden increase.

- Working in the course of nature, the temperature of the bearing should be below 35℃,the max. temperature should not to be over 70℃,if the temperature is over 70℃,please shut down the machine at once, find out the causes and eliminate them.

- Before shutting up the machine, firstly stop feeding the materials, when all the materials being crushed in the crushing cavity are discharged, then the electric motor can be shuttled down.

- 5. In operating jaw crushers, if the crushers are stopped by clogging of the materials in the crushing cavity, shut down the electric motor immediately, eliminate he materials, then you can restart electric motor

6. The jaw plates can be reversed end for end when one end is wore out.

Lubricating

- Often adverting and lubricating the wearing faces in time, assuring the machine working normally and prolonging the wearing life of the crushers.

- The greases which adopt by our machine is on the basis of the location and temperature, we usually choose calcium-group, atrium-group or calcium and atrium-group greases.

- The grease added to the bearing seat is 50-70% of its volume and must be replaced every three months, and the track of the roller bearing is carefully cleaned with clean gasoline or kerosene when the oil is changed.

- Toggle mechanism and gliding contacts must be lubricated with grease before starting the jaw crusher.

Wearing parts names

- Liners  2.Side Plates  3.Fixed Jaw Plates  4.Movable Jaw plates                     5. Sliding Blocks

Safety Rules

- The operator must be trained with safety technology education.

- Ban to look into the machine from its top when machine works.

- Ban to do any adjusting cleanings or examining and repairing when machine works.

- Ban to use hands directly to transit or remove the rocks in the crushing cavity or on the feeder.

- The electric equipment should be earthling and put the electrical wire into the isolated  pipes.